EE-384: Digital Signal Processing Spring 2020 Laboratory 2: Discrete-time Signals in MATLAB Instructor: Mr Ammar Naseer EE UET New Campus

# Aims

We begin with the concepts of signals in discrete time. A number of important types of signals and their operations are introduced. The emphasis in this chapter is on the representations and implementation of signals using MATLAB.

Pre-Lab:

## Discrete time Signals

Signals are broadly classified into analog and discrete signals. An analog signal will be denoted by x(t), in which the variable t can represent any physical quantity, but we will assume that it represents time in seconds. A discrete signal will be denoted by x(n), in which the variable n is integer-valued and represents discrete instances in time. Therefore it is also called a discrete-time signal, which is a number sequence and will be denoted by one of the following notations.

where the up-arrow indicates the sample at n = 0.

In MATLAB we can represent a finite-duration sequence by a row vector of appropriate values. However, such a vector does not have any information about sample position n. Therefore a correct representation of x(n) would require two vectors, one each for x and n. For example, a sequence x(n) = [2 1 -1 0 1 4 3 7]  can be represented in MATLAB by

Generally, we will use the x-vector representation alone when the sample position information is not required or when such information is trivial (e.g. when the sequence begins at n = 0). An arbitrary infinite-duration sequence cannot be represented in MATLAB due to the finite memory limitations.

Page 1

## Type of Sequences

2.A We use several elementary sequences in digital signal processing for analysis purposes. Their definitions and MATLAB representations follow.

### Unit Sample Sequence

In MATLAB the function zeros (1,N) generates a row vector of N zeros, which can be used to implement

(n) over a finite interval. However, the logical relation n==0 is an elegant way of implementing (n). For example, to implement

over the n1 < n0 < n2 interval, we will use the following MATLAB function

For example, to generate  ; -5 ≤ n ≤ 5, we will need the following MATLAB script:

>> [x,n] = impseq(-5,5,2);

>> stem(n,x)

### Unit Step Sequence

In MATLAB the function ones(1,N) generates a row vector of N ones. It can be used to generate u(n) over a  finite interval. Once again an elegant approach is to use the logical relation n>=0. To implement

over the n1 < n0 < n2 interval, we will use the following MATLAB function

For example, to generate ; -5 ≤ n ≤ 5, we will need the following MATLAB script:

>> [x,n] = stepseq(-5,5,-1);

>> stem(n,x)

### Real-valued exponential sequence

In MATLAB an array operator "." is required to implement a real exponential sequence.

For example, to generate x(n) = (0.9)n; 0 ≤ n ≤ 10, we will need the following MATLAB script:

>> n = [0:10]; x = (0.9).^n;

>> stem(n,x)

### Complex-valued exponential sequence

Where  produces an attenuation (if < 0) or amplification (if > 0) and  is the frequency in radians. A MATLAB function exp is used to generate exponential sequences.

For example, to generate x(n) = exp[(2+j3)n], 0 ≤ n ≤ 10, we will need the following MATLAB script:

>> n = [0:10];x = exp((2+3j)*n);

>> stem(n,x)

### Sinusoidal sequence

where A is an amplitude and  is the phase in radians. A MATLAB function cos (or sin) is used to generate sinusoidal sequences. For example, to generate .

0<n<10, we will need the following MATLAB script:

>> n = [0:10]; x = 3*cos(0.1*pi*n+pi/3) + 2*sin(0.5*pi*n);

>> stem(n,x)

### Periodic Sequence

A sequence x(n) is periodic if x(n) = x(n+N); n. The smallest integer N that satisfies this relation is called the fundamental period. We will use (n) to denote a periodic sequence. To generate P periods of  (n) from one period {x(n), 0<n<N1}, we can copy x(n) P times:

>> xtilde = [x,x,...,x];

But an elegant approach is to use MATLABs powerful indexing capabilities. First we generate a matrix containing P rows of x(n) values. Then we can concatenate P rows into a long row vector using the construct (:). However, this construct works only on columns. Hence we will have to use the matrix transposition operator ' to provide the same effect on rows.

>> xtilde = x' * ones(1,P); % P columns of x; x is a row vector

>> xtilde = xtilde(:); % long column vector

>> xtilde = xtilde'; % long row vector

Note that the last two lines can be combined into one for compact coding.

Operation on sequence

2.B Here we briefly describe basic sequence operations and their MATLAB equivalents.

1. Signal addition: This is a sample-by-sample addition given by

It is implemented in MATLAB by the arithmetic operator +. However, the lengths of x1(n) and x2(n) must be the same. If sequences are of unequal lengths, or if the sample positions are different for equal-length sequences, then we cannot directly use the operator +. We have to first augment x1(n) and x2(n) so that they have the same position vector n (and hence the same length). This requires careful attention to MATLABs indexing operations. In particular, logical operation of intersection &, relational operations like ;= and ==, and the find function are required to make x1(n) and x2(n) of equal length. The following function, called the sigadd function, demonstrates these operations.

For example, to generate  +

; -5 ≤ n ≤ 5, we will need the following

MATLAB script:

>> [x1,n1] = impseq(-5,5,4);

>> [x2,n2] = impseq(-5,5,-2);

>> [x,n] = sigadd(x1,n1,x2,n2);

>> stem(n,x)

- Signal Multiplications: This is a sample-by-sample (or "dot") multiplication) given by

It is implemented in MATLAB by the array operator ” * ”. Once again, the similar restrictions apply for the “ .* operator as for the + operator”. Therefore we have developed the sigmult function, which is similar to the sigadd function.

For example, to generate  *

; -5 ≤ n ≤ 5, we will need the following

MATLAB script:

>> [x1,n1] = impseq(-5,5,4);

>> [x2,n2] = impseq(-5,5,-2);

>> [x,n] = sigmult(x1,n1,x2,n2);

>> stem(n,x)

- Shifting In this operation, each sample of x(n) is shifted by an amount k to obtain a shifted sequence y(n).

If we let m = nk, then n = m + k and the above operation is given by

Hence this operation has no effect on the vector x, but the vector n is changed by adding k to each element. This is shown in the function sigshift.

For example, to generate  ; -3 ≤ n ≤ 7, we can also write the following MATLAB script:

>> [x,n] = impseq(-5,5,0);

>> stem(n,x)

>> [y,n] = sigshift(x,n,2);

>> stem(n,y)

- Folding In this operation each sample of x(n) is flipped around n = 0 to obtain a folded sequence y(n).

In MATLAB this operation is implemented by fliplr(x) function for sample values and by -fliplr(n) function for sample positions as shown in the sigfold function.

For example, to generate  ; -5 ≤ n ≤ 5, we can also write the following MATLAB script:

>> [x,n] = impseq(-5,5,2);

>> stem(n,x)

>> [y,n] = sigfold(x,n);

>> stem(n,y)

- Sample summation. This operation differs from signal addition operation. It adds all sample values of x(n) between n1 and n2.

It is implemented by the sum(x(n1:n2)) function.

CODE:

>> [x,n] = stepseq (5,0,10);

>> sum(x(2:7))

ans =  2

>>

- Sample Product: This operation also differs from signal multiplication operation. It multiplies all sample values of x(n) between n1 and n2.

It is implemented by the prod(x(n1:n2)) function.

CODE:

>> c=[0 1 2 3 4 5]

>> prod(c(2:5))

ans =  24

>>

- Signal energy. The energy of a sequence x(n) is given by

where superscript * denotes the operation of complex conjugation. The energy of a finite-duration sequence x(n) can be computed in MATLAB using

CODE:

# >> x=[1 2 3 4 5];

# >> Ex=sum(x.*conj(x));

# >> Ex

# Ex =  55

# >> Ex=sum(abs(x).^2)

# Ex =  55

# >>

# Main Lab

3A: Generate and plot each of the following sequences over the indicated interval.

- ,

CODE:

>> n=[-8:0];

>> x=3*impseq(-8,0,3)-impseq(-8,0,6);

>> stem(n,x)

- ,

CODE:

>>  n=[0:20];

>> x1=n.*(stepseq(0,0,20)-stepseq(10,0,20));

>> x2=10*exp(-0.3*(n-10)).*(stepseq(10,0,20)-stepseq(20,0,20));

>> [x,n1]=sigadd(x1,n,x2,n);

>> subplot(3,1,1);

>> stem(n,x1);

>> axis([-10 20 -10 10])

>> grid on;

>> subplot(3,1,2);

>> stem(n,x2);

>> grid on;

>> axis([-10 20 -10 10]);

>> subplot(3,1,3);

>> stem(n1,x);

>> grid on;

>> axis([-10 20 -10 10]);

CODE:

>> x=[5 4 3 2 1];

>> n=0:length(x)-1;

>> subplot(2,1,1);

>> stem(n,x);

>> title("Orignal Signal");

>>  x1=[x,x,x,x];

>> n=-10:9;

>> stem(n,x1);

>> title("Periodeic Signal");

3B: Determine and plot the following sequences

CODE:

>> n=[-2,-1,0,1,2,3,4,5,6,7,8,9,10];

>> x=[-1,2,3,4,5,6,7,8,5,4,3,-2,1];

>>[y1,n1]=sigshift(2*x,n,5);

>>[y2,n2]=sigshift(-3*x,n,-4);

>>[x1,n3]=sigadd(y1,n1,y2,n2)

CODE:

>> [y1,n1]=sigshift(4*x,-n,-5);

>> [y2,n2]=sigshift(-3*x,n,-2);

>> [x1,n3]=sigadd(y1,n1,y2,n2);

>>  subplot(3,1,1)

>>  stem(n1,y1)

>> subplot(3,1,2)

>> stem(n2,y2)

>>  subplot(3,1,3)

>> stem(n3,x1)

3C: Generate the complex-valued signal

and plot its magnitude, phase, the real part, and the imaginary part in four separate subplots.

CODE:

>> n=[-10:10];

>> x=exp((0.1+0.3j)*n);

>> subplot(4,1,1);

>> stem(n,real(x));

>> xlabel('Time Samples');

>> ylabel('Real Value');

>> subplot(4,1,2);

>> stem(n,imag(x));

>> xlabel('Time Samples');

>> ylabel('Imaginary Value');

>> subplot(4,1,3);

>> stem(n,abs(x));

>> xlabel('Time Samples');

>>  ylabel('Magnitude Value');

>> subplot(4,1,4);

>> stem(n,angle(x));

>> xlabel('Time Samples');

>> ylabel('Angle Value');

>>